# Activity 1 : Self test sheet

## Aim

To ensure that the robot is built and wired correctly and that there are no hardware faults in the robot.

## Preparation

Add TWO wire links to the robot as follows

Front link

Back link

The links are provided in a pack in your teacher’s kit.  The pack is labelled “wire links”.

## Running the test

Put a  or a  in the last column if the test was successful or failed.

| Test | Action | Comments | or  |
| --- | --- | --- | --- |
| 1. | Place robot on the floor and switch on. | Display should shows the message tESt8 |  |
|  | Press any switch to continue to the next test. |  | ------ |
| 2. | As each LED comes ON press the switch below | LED A comes On  press SW-A LED B comes On  press SW-B LED C comes On  press SW-C LED D comes On  press SW-D Tests the four LEDs and the four switches |  |
| 3. | Ensure robot is on the floor then press switch A | Robot should move forward for 5 seconds |  |
| 4. | Press switch A | Robot should turn left for 3 seconds |  |
|  |  | Display will now show the message SEt POtS | ---- |
| 5. | Set all 3 POTs to point to the batteries. Then press switch A | If successful the display will show tESt_End If there is a problem the display will show Err_Pot_1 or 2 or 3 |  |

If five  ‘s then robot works and is correctly wired.  If any  ‘s then get copy of diagnostic sheet from your teacher.