# Activity 1 : Self test diagnostic sheet

## Diagnosing problems based on the robot failing the self-test

You have a robot that has failed the self test activity.  The following will help to find out the problem.

- FIVE  ‘s means all is fine and the motor wiring is correct.

  - Now remove the wire links and store in your kit box.  You can always rerun the test if you feel that the robot is not working correctly.

- Test 1 fail

  - Check batteries are correctly inserted and rerun the test.

  - Try different set of batteries and rerun test.

    - If successful recharge batteries.

- Test 2 fail

  - Faults in switches or LEDs

    - Use spare robot and return original for repair.

- Test 3 fail

  - Like to be a wiring problem.

    - Make sure the LEFT motor is wired to the left connector.

    - Make sure the RIGHT motor is wired to the right connector

    - BLACK wires connect to connector-1

    - RED wires connect to connector-2

Black wire from LEFT motor

Black wire from RIGHT motor

Red wire from LEFT motor

Red wire from RIGHT motor

- Test 4 fail

  - Again, likely to be a wiring problem.  See Test 3 fail information.

- Test 5 fail

  - Check that POTS point exactly to the battery as in following picture, then rerun test.

  - A similar test failure means that there is a problem with the robot hardware.

    - Use spare robot and return original for repair