# Activity 1 : Test chart 1

- Disconnect switch controller and do the following

Power ON

Power OFF

Check batteries for correct insertion

Power ON

FAULT

Return unit to HW

Power OFF

Recharge batteries

Power-ON

Yes

Yes

Yes

No

No

No

No

Power OFF

Power ON

Yes

FAULT

Return unit to HW

Power light ON ?

Yes

Power light ON ?

Power light ON ?

Buzzer sound ?

Buzzer sound ?

No

Yes

OK

Continue

Go to fault chart 2

# Activity 1 : Test chart 2

If you have reached this point then Robokid has been successfully powered on.  Next task is to run a small test and observe the effects.

Run test

Use the following switch sequence

Press Switch A

Press Switch D

Press Switch A twice

To get to mode shown as “A0” on the display

Place robot on the floor and press switch A

Observe results

The following table gives a list of all the possible results

| Case | Effect observed | Fault | Fixes for fault |
| --- | --- | --- | --- |
| 1 | Robot drives forward | No | No fault, robot is correctly wired. |
| 2 | Robot drives backward | Yes | For each motor, black and red wires need to be swapped. BLACK should go to connector 1 and RED wire should go to connector 2 |
| 3 | Robot spins sharply to the LEFT. | Yes | Wiring for RIGHT motor is correct. Rewire LEFT motor making sure that black wire is in connector 1 and red wire is in connector 2 |
| 4 | Robot spins sharply to the RIGHT. | Yes | Wiring for LEFT motor is correct. Rewire RIGHT motor making sure that black wire is in connector 1 and red wire is in connector 2 |
| 5 | Spins to LEFT but with LEFT wheel not moving | Yes | Wiring for RIGHT motor is correct. Rewire LEFT motor making sure that black wire is in connector 1 and red wire is in connector 2. |
| 6 | Spins to RIGHT but with RIGHT wheel not moving | Yes | Wiring for LEFT motor is correct. Rewire RIGHT motor making sure that black wire is in connector 1 and red wire is in connector 2 |
| 7 | Robot does not move | Yes | Redo wiring for both motors making sure that black wire is in connector 1 and red wire is in connector 2 |