### Activity 7 worksheet : line bump

Experiment 1

- Create a continuous line on a sheet of paper using black tape.  Make sure there are no gaps.

- Use the robot without the switch controller.

- Place your robot in centre of area and power-on.  Select mode “b0”.  Switch sequence is “A_2D_2A”.

- Now press SW-A to start.  If all is OK the robot will bump its way around the area set by the tape.  It will stop after 2 minutes or if you press and hold  SW-A  or SW-C for a few seconds.

- Insert felt-tip pen and press SW-A again. What kind of pattern has the robot drawn?

Experiment 2

- Now experiment with the POT settings

POT-1  :: Sets the amount of reverse when the robot detects the black tape

POT-2  :: Sets the amount of spin after the robot has reversed

POT-3  :: Sets the speed difference between the two wheels.  A greater difference means that it moves more in a tighter curve.

Remember to press SW-B to get the robot to read the POT settings

- Swap pens with another group to get a multi-coloured picture.

Experiment 3 – MOUSE mode

- Turn paper over and make a new shape for experiment 3.

- Place your robot in centre of area and power-on.  Select mode “b1”.  Switch sequence is “A_2D_A_D_A”

- Now press SW-A to start.  If all is OK the robot will bump its way around the area set by the tape.  It will stop after 2 minutes or if you press and hold  SW-A  or SW-C for a few seconds.

- How is this different from bump mode in experiment 1?

- IMPORTANT : The POTs have no effect in this mode “b1”.

Experiment 4

- Design an experiment to see if you can use other black materials in place of the black tape. Try

Black felt-tip pen, black biro, black photocopier ink, black poster paint, others????

(POSTER PAINT MUST BE DRY BEFORE RUNNING THE ROBOT)

- Discuss the results with your teacher.